CPE301 – SPRING 2019

# Design Assignment 3A

Student Name: Mat Tomljenovic

Student #: 1012254710

Student Email: tomljeno@unlv.nevada.edu

Primary Github address: https://github.com/matcroatia/

Directory: https://github.com/matcroatia/DA3A

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

List of components: Internal resistor, Pin C PORT B, Atmega328P

Pins: PB02 and PC02, LED1 and SW1 switch.

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* Assignment 3.c

*

* Created: 3/27/2019 6:20:28 PM

* Author : Mat Tomljenovic

*/

#define F_CPU 8000000UL

#include <avr/io.h>

#include <util/delay.h>

#include <stdio.h>

#include <avr/interrupt.h>

#define BAUDRATE 9600

#define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 16UL))) - 1)

void USART_init( unsigned int ubrr );	//function for unsigned integer

void USART_tx_string( char *data );		//function for char data

volatile int Count;						//integer variable count

char outs[60];							//number of character to be allowed

int n;									//integer declaration

float PI;								//floating point number declaration

char str[] = "The Sleeper Has Awakened!"; //string to be displayed

char empty[] = " ";						//declaration to print blank space

/* MAIN FUNCTION */

int main(void)

{

Count  = 0;								//count used for timer

TIMSK0 |= (1 << TOIE0);					//interrupt used when timer overflows

sei ();									//enable interrupts

TCCR0A = 0x00;							//enable normal mode

TCCR0B = 0x05;							//set pre-scaler to 1024

USART_init(BAUD_PRESCALLER);			//initiate baud pre-scaler

USART_tx_string("Connected!\r\n");		//displays if serial is connected to output

while(1);

}

/* INIT USART (RS-232)  */

void USART_init( unsigned int ubrr )

{

UBRR0H = (unsigned char)(ubrr>>8);		//baud rate register for high byte

UBRR0L = (unsigned char)ubrr;			//baud rate register for low byte

UCSR0B = (1 << TXEN0);					//Enable RX, TX & RX interrupt

UCSR0C = (3 << UCSZ00);					//asynchronous 8 N 1

}

/* SEND A STRING TO THE RS-232 */

void USART_tx_string( char *data )

{

while ((*data != '\0'))					//wait while data transmission is ready

{

while (!(UCSR0A & (1 <<UDRE0)));

UDR0 = *data;						//store data into register UDR0

data++;

}

}

ISR (TIMER0_OVF_vect)

{

while (Count < 61)						//while loop to count

{

if ((TIFR0 & 0x01) == 1)			//sets overflow flag to 1

{

TIFR0 = 0x01;

Count++;						//increments count

}

}

if (Count > 60)							//while loop to count above 60

{

n = rand();							//integer used to generate random number

PI = 3.1415;						//declaration for floating point decimal

dtostrf(PI, 6, 6, outs);			//print PI and declare how many decimal points print

USART_tx_string(outs);				//prints declared string

USART_tx_string(empty);				//print a blank space

snprintf(outs, sizeof(outs), "%3d\r\n" , n);	//converts random number to integer

USART_tx_string(outs);				//prints out PI

USART_tx_string(empty);				//prints out blank space

USART_tx_string(str);				//prints out random number

USART_tx_string(empty);				//prints out blank space

Count = 0;							//resets counter back to 0

}

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

Same as above.

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/miiUzg9G94Y

- GITHUB LINK OF THIS DA

https://github.com/matcroatia/DA3A

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

NAME OF THE STUDENT